Saltmarsh Energy — fatigue and staged-trial evidence

GLASSMERE TERMINALS  |  BOARD BRIEF

Prepared for Glassmere Terminals contracting and legal review | 06 March 2026

# Measured accelerated-fatigue result

Saltmarsh tested 12 alternate elastic rail clips at 1.5× the nominal service load for 2.0 million cycles. All 12 completed the test without fracture or loss of retention. This is evidence of performance under the stated accelerated condition; it is not proof of equivalent field life.

| Measure | Observed result | Reference |
| --- | --- | --- |
| Population | 12 clips | FAT-01 |
| Applied condition | 1.5× nominal service load; 2.0 million cycles | FAT-01 |
| Outcome | 12/12 completed; no fracture or loss of retention | FAT-02 |

# Service-history comparison

The alternate has 7.3 years of relevant in-track history; the incumbent has 31.8 years. The shorter alternate record remains a material uncertainty for an approval decision.

| Item | Duration | Reference |
| --- | --- | --- |
| Saltmarsh alternate | 7.3 years | HIST-01 |
| Current incumbent | 31.8 years | HIST-01 |

# Proposed staged Glassmere trial

The proposal is a bounded path to gather field evidence while retaining escalation controls.

| Stage / timing | Control point | Acceptance / escalation | Owner | Reference |
| --- | --- | --- | --- | --- |
| Stage 1 — 0–30 days | Baseline fit-up inspection; photograph rail seat and clip position | Accept if fit-up checklist complete; escalate any damage or misfit | Lucia Jelinek | TRIAL-01 |
| Stage 2 — 31–90 days | 30-day and 90-day inspections; record retention and visible wear | Continue if no loss of retention and wear record is complete; stop and review on deviation | Mateo Ainsworth | TRIAL-02 |
| Stage 3 — 91–180 days | End-of-trial condition review and evidence pack | Recommend alternate listing only after review of records and unresolved verification sample | Mateo Ainsworth / Lucia Jelinek | TRIAL-03 |

# Commercial context

Supplied unit rate: USD 18.47 per clip. The rate is for the clip supply only; installation, inspection, and trial administration are outside this quoted unit rate. (COMM-01)

# Limitations

Accelerated testing does not establish equivalent field life. Service-history data are durations, not a reliability guarantee. Trial timing and gates above are proposed controls, subject to Glassmere confirmation.