DUNMARROW PROVISIONS | PALLET PUT-AWAY LOCATION STANDARD

Controlled draft • Owner: Inventory Control • Wisconsin warehouse • Rev 0 | 30 Jun 2025

Purpose: a receiver records facts and applies the matrix before placing a pallet. This is an inventory-control record—not a certification of legal compliance, sprinkler adequacy, or rack engineering.

Basis, definitions & governing rule

Jurisdiction/basis: Federal OSHA general-industry requirements for a private-sector Wisconsin warehouse, as of June 30, 2025. Trace: 29 CFR 1910.176(a) (clear aisles, safe clearances, permanent aisle marking where mechanical handling equipment is used) and 29 CFR 1910.176(b) (stored material stable and secure against sliding, collapse, or falling). OSHA Subpart N does not provide one universal aisle width, stack height, or sprinkler distance.

Sprinkler clearance = vertical separation from the projected top of the load after placement to sprinkler/deflector or other protected reference, using the current property-insurer/fire-protection schedule. Aisle/travel clearance = usable passage and equipment travel space around the pallet, using the site/equipment minimum. Never enter one in the other field.

All applicable conditions are cumulative. Law is the floor; the rack manufacturer plaque controls structural loading within its stated unit/scope; the insurer schedule controls its fire-protection requirement; WMS rules control inventory eligibility. The most restrictive applicable constraint controls its dimension. If sources conflict or a required value is missing, HOLD / REFER—no receiver waiver.

Put-away decision matrix — follow in order; judge the condition after placement

| STEP / GATE | RECORD & TEST | IF NOT PASS / NOT KNOWN |
| --- | --- | --- |
| 1 ¦ Identify | Item/SKU, receipt or pallet ID, quantity; proposed location: ☐ floor area ____ ☐ rack ID ____ bay ____ level ____; WMS: ☐ fixed ☐ fluid; item-location eligibility ☐ pass ☐ fail ☐ unknown. | Unknown identity, location, or WMS status → HOLD / REFER. |
| 2 ¦ Weight evidence | Gross weight ____ [unit]; source ☐ scale ☐ supplier document ☐ verified item master ☐ other ____; date/source ID ____ . | Weight unavailable or unverified → HOLD. Never assume nominal weight. |
| 3 ¦ Fit & dimensions | Pallet H ____; footprint L×W ____; overhang/encroachment ____; projected top after placement ____ . Confirm fit, no overhang that creates a hazard, and condition-based height controls. | Does not fit, overhang/height creates hazard, or stack would be unstable → REJECT / alternate. |
| 4 ¦ Stability & product | Pallet/load ☐ sound ☐ damaged; load ☐ stable/secured ☐ leaning ☐ shifting ☐ unsecured. Product/location compatibility and segregation ☐ pass ☐ fail ☐ unknown (temperature, contamination, product class or site restriction). | Damage, leaning, shifting, unsecured load, or incompatibility → HOLD / REJECT; correct or refer. |
| 5 ¦ Rack capacity (rack only) | Read applicable plaque/manufacturer source: limit ____ [unit and scope] for rack ____ / bay ____ / level ____. Existing relevant level load ____ + incoming gross load ____ = projected post-placement load ____ . Compare in same units/scope. A beam/level limit is not automatically a per-pallet allowance; do not divide or reinterpret it. | Capacity/source cannot be verified, units/scope unclear, or projected load exceeds limit → HOLD / REFER to manufacturer or qualified structural authority. |
| 6 ¦ Fire & travel | Required sprinkler clearance ____ [unit], source/edition ____ (insurer schedule; editable site value). Projected actual sprinkler clearance after placement ____ . Aisle/travel minimum ____ [site/equipment value], measured usable ____ . | Required value missing, actual < required, aisle/passage obstructed, or unsafe clearance → HOLD / alternate. Do not treat aisle clearance as sprinkler clearance. |
| 7 ¦ Access & rack condition | After placement: exits/exit access ☐ clear; emergency/fire equipment ☐ accessible; electrical working space ☐ clear; permanent aisle marking ☐ present where powered equipment is used. Rack ☐ no visible damage/missing parts/out-of-service tag ☐ concern. | Any obstruction or rack concern → HOLD, isolate/tag location, and notify EHS/fire function or authorized rack authority. |
| 8 ¦ Outcome | Only if every applicable input is resolved and passes: ☐ APPROVE FOR PUT-AWAY. Otherwise record ☐ HOLD / REFER ☐ REJECT / ALTERNATE LOCATION. Record reason and disposition on page 3. | WMS assignment or open space is never safety, fire, or structural approval. |

Exception route: only the responsible EHS/fire-protection specialist or rack manufacturer/qualified structural authority may document an exception or alternate control. Bohdan (inventory) records observations, source IDs, and the matrix outcome; he may not waive a requirement or certify legal, fire-system, or structural compliance.

STORAGE-LOCATION APPROVAL FORM

Complete one form per pallet / proposed location • Attach scale ticket, source photo, or WMS printout when available

A. Receipt, pallet & proposed destination

| Decision date | ____ / ____ / ______ | Receiver / preparer (name or ID) | ________________________ |
| --- | --- | --- | --- |
| Item / SKU | ________________________ | Pallet / receipt / lot ID | ________________________ |
| Quantity + UOM | ________________________ | Product restrictions / segregation | ☐ pass ☐ fail ☐ N/A Notes: ______ |
| Location type | ☐ floor ☐ rack | Rack ID / bay / level or floor area | ________________________ |
| WMS location | ________________________ | WMS status | ☐ fixed ☐ fluid Eligibility ☐ pass ☐ fail ☐ unknown |

B. Load facts and dimensional fit

| Pallet dimensions | H ____ × L ____ × W ____ [units] | Overhang / projected top | ________________________ |
| --- | --- | --- | --- |
| Gross weight | ________ [unit] | Weight verification source / ID | ☐ scale ☐ supplier ☐ item master ☐ other ______ |
| Pallet/load condition | ☐ sound ☐ damaged ☐ stable ☐ leaning/shifting ☐ secured | Fit after placement | ☐ fits ☐ overhang/height concern Notes: ______ |

C. Rack source and projected loading (complete for rack storage)

| Rack / plaque source | Rack ID ____ Plaque/photo/doc ID ____ | Manufacturer / revision | ________________________ |
| --- | --- | --- | --- |
| Posted limit (do not invent) | ________ [unit] Scope: ☐ beam ☐ level ☐ other ____ | Existing relevant level load | ________ [same unit/scope] |
| Incoming load | ________ [same unit/scope] | Projected post-placement load | ________ [same unit/scope] |
| Capacity comparison | ☐ projected ≤ verified limit ☐ exceeds ☐ cannot verify | Rack condition | ☐ no visible concern ☐ damage/missing/out-of-service |

STORAGE-LOCATION APPROVAL FORM — CONTINUED

Use the matrix outcome; blank or conflicting source data is not an approval

D. Fire-protection and access checks (post-placement)

| Insurer schedule source | Schedule ID / edition / date: __________________ | Required sprinkler clearance | ________ [unit] VERIFIED ☐ |
| --- | --- | --- | --- |
| Projected actual sprinkler clearance | ________ [unit] Measured after placement | Sprinkler test | ☐ actual ≥ required ☐ less ☐ unknown |
| Aisle / travel minimum | ________ [unit] Site/equipment source: ______ | Usable aisle/travel after placement | ________ [unit] ☐ clear ☐ obstructed |
| Egress / protected access | Exits & exit access ☐ clear ☐ blocked | Emergency/fire equipment + electrical working space | ☐ accessible/clear ☐ blocked |
| Aisle marking | Permanent aisle marking where mechanical handling equipment is used ☐ yes ☐ no ☐ N/A | Notes / photo reference | ________________________________ |

E. Decision, reason & controlled review

| Final status | ☐ APPROVE FOR PUT-AWAY (all applicable gates pass) ☐ HOLD / REFER ☐ REJECT / ALTERNATE LOCATION |
| --- | --- |
| Reason if not approved | ☐ missing weight ☐ capacity/source ☐ sprinkler clearance ☐ aisle/access ☐ unstable/damaged load ☐ rack concern ☐ incompatibility ☐ source conflict ☐ other ______ |
| Disposition / alternate | New location or action: ______________________________________________ Ticket / referral ID: __________________ |
| Inventory verification | I attest only that the recorded identity, measurements, observations, source references, and matrix completion are accurate to the best of my knowledge. I do not certify legal compliance, sprinkler adequacy, or rack engineering. Name/ID: __________ Date: ______ |
| Triggered specialist review | EHS / fire-protection reviewer (if required): __________ Status/date: __________ Rack manufacturer / qualified structural authority (if required): __________ Status/date: __________ |

Quick source trail

OSHA: 29 CFR 1910.176(a) and (b), Materials Handling and Storage (accessed/as-of 30 Jun 2025); OSHA Subpart N is not a universal aisle-width, stack-height, or sprinkler-clearance table. Site records to verify before use: current rack manufacturer plaque/engineering information; current property-insurer or fire-protection schedule; WMS location master and item restrictions.

Record retention / escalation: file with receiving record; isolate a concerning rack or obstruction and notify the authorized function. A WMS “available” or “assigned” location never overrides this form.